# P30+LSTM Prototype Development Summary

A prototype autonomous collision-avoidance framework was developed to investigate the suitability of the P3O + LSTM approach proposed by Mu et al[1]. for CubeSat operations. The prototype used conjunction-event data from the ESA Kelvins Collision Avoidance Challenge dataset and explored how reinforcement learning could be combined with temporal state representation and an independent safety layer.

# Prototype Architecture

The prototype was formulated as a Partially Observable Markov Decision Process (POMDP), in which sequential Conjunction Data Messages (CDMs) provide partial observations of an evolving conjunction event. An LSTM-based temporal representation was investigated to capture dependencies between successive CDMs and maintain a belief-state estimate of the encounter. A PPO-based decision policy was then used to select collision-avoidance manoeuvres from a discrete action set, while an independent safety layer enforced CubeSat mission constraints, including ΔV limitations.

## POMDP Diagram

Conjunction Data Messages (CDMs)

↓

Observation Space

↓

LSTM Temporal Encoder

↓

Belief-State Estimate

↓

PPO Policy

↓

Candidate Manoeuvre

↓

Safety Layer

↓

Approved Manoeuvre

↓

Risk & Fuel Evaluation.

## CubeSat Assumptions

To reflect realistic operational constraints, the prototype assumed a representative CubeSat platform:

| Parameter | Value |
| --- | --- |
| Spacecraft Mass | 10 kg |
| Thruster | 1 mN |
| Specific Impulse | 1500 s |
| ΔV Budget | 25 m/s |

Propellant consumption was estimated using the Tsiolkovsky rocket equation:

m_fuel=m_01-e-ΔVI_spg_0

where:

- m_fuel= propellant mass consumed

- m_0= spacecraft mass

- ΔV= manoeuvre magnitude

- I_sp= specific impulse

- g_0= standard gravity

This allowed each manoeuvre recommendation to be evaluated against realistic CubeSat resource constraints.

## Manoeuvre Options

The PPO agent selected from seven possible actions:

| Action | Description |
| --- | --- |
| 0 | No manoeuvre |
| 1 | Small positive along-track burn |
| 2 | Small negative along-track burn |
| 3 | Medium positive along-track burn |
| 4 | Medium negative along-track burn |
| 5 | Small positive cross-track burn |
| 6 | Small negative cross-track burn |

- Along-track manoeuvres

Along-track burns accelerate or decelerate the spacecraft along its orbital path. This changes the arrival time at the conjunction location and is a common technique for reducing collision risk.

- Cross-track manoeuvres

Cross-track burns move the spacecraft above or below the original orbital plane, creating additional separation at closest approach.

The medium manoeuvres corresponded to larger ΔV expenditures than the small manoeuvres and therefore provide greater risk reduction at a higher fuel cost.

## Safety Layer

An independent safety layer was included to provide an additional level of assurance before manoeuvres are executed.

The safety layer:

- Reviews PPO manoeuvre recommendations

- Enforces ΔV constraints

- Prevents excessive manoeuvre expenditure

- Acts independently of the learned policy

This ensures that autonomous decisions remain subject to predefined mission limits rather than having unrestricted authority.

# Evaluation Method

The framework was evaluated using representative conjunction events selected from the Kelvins dataset, including low-, medium- and high-risk scenarios. The reward function was designed to balance multiple objectives, including collision-risk reduction, miss distance and ΔV expenditure.

Performance was assessed using:

- Estimated collision-risk reduction

- Fuel consumption

- Total ΔV expenditure

- Manoeuvre-selection behaviour

- Cumulative reward

# Results

The prototype was evaluated using eight representative conjunction events from the ESA Kelvins Collision Avoidance Challenge dataset, covering low-risk, medium-risk, high-risk and challenge-case scenarios.

The initial untrained baseline consistently selected no manoeuvre and therefore achieved no meaningful collision-risk reduction.After PPO training, the policy learned non-trivial manoeuvre-selection behaviour and predominantly selected medium-magnitude collision-avoidance manoeuvres for more challenging conjunction events.

Table I: P30 + LSTM Prototype Results

| S/N | Event Type | Event ID | Risk Before | Risk After | Risk Reduction | Dominant Action | Total ΔV (m/s) | Fuel Used (g) | Total Reward |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Low Risk | 11953 | -4.516 | -3.613 | 0.903 | -Along-track Medium | 8.5 | 5.775 | 7,517 |
| 2 | Low Risk | 6792 | -4.564 | -3.651 | 0.913 | -Along-track Medium | 10.0 | 6.793 | 8,739 |
| 3 | Medium Risk | 1 | -9.316 | -7.453 | 1.863 | -Along-track Medium | 2.0 | 1.359 | 3,339 |
| 4 | Medium Risk | 25 | -9.537 | -7.629 | 1.907 | -Along-track Medium | 3.5 | 2.378 | 6,372 |
| 5 | High Risk | 5 | -30.000 | -24.000 | 6.000 | +Along-track Medium | 10.0 | 6.793 | 24,049 |
| 6 | High Risk | 13 | -30.000 | -24.000 | 6.000 | -Along-track Medium | 10.5 | 7.133 | 45,579 |
| 7 | Challenge Case | 12 | -30.000 | -24.000 | 6.000 | -Along-track Medium | 6.5 | 4.416 | 23,088 |
| 8 | Challenge Case | 21 | -30.000 | -24.000 | 6.000 | -Along-track Medium | 10.5 | 7.133 | 61,364 |

The results show that the trained PPO policy consistently selected manoeuvres that reduced the estimated collision risk across all evaluated scenarios. Risk reductions were generally larger for the high-risk and challenge-case events, indicating that the policy allocated more manoeuvre effort when the threat level increased.

The dominant action selected was a medium along-track manoeuvre, suggesting that the policy learned that changing the spacecraft's timing along its orbit was often the most effective method of reducing collision risk within the available fuel budget. Along-track manoeuvres either accelerate or decelerate the spacecraft, altering its arrival time at the conjunction point and increasing separation from the threatening object.

Fuel consumption remained relatively low across all scenarios, ranging from approximately 1.4 g to 7.1 g, while all manoeuvres remained well within the assumed 25 m/s CubeSat ΔV budget. This demonstrates that the framework was able to balance collision-risk reduction against propellant expenditure.

Overall, the prototype demonstrated successful learning of non-trivial collision-avoidance behaviour, with the PPO policy adapting manoeuvre recommendations according to conjunction severity while respecting representative CubeSat mission constraints.

# Limitations

The prototype was intended as a feasibility study rather than an operational flight solution. Thus, several simplifications were made:

- Post-manoeuvre risk was estimated using an analytical approximation rather than full orbital propagation.

- The implementation evaluated individual conjunction events independently.

- Simultaneous multi-threat scenarios were not explicitly modelled.

- Validation was performed on historical conjunction-event data.

Further work would require high-fidelity orbital propagation, uncertainty propagation, verification on unseen conjunction scenarios and testing on representative flight hardware.

# Conclusion

The prototype demonstrated that key concepts from the P3O + LSTM architecture can be successfully integrated into a CubeSat collision-avoidance framework. The combination of sequential conjunction information, reinforcement-learning-based decision making, fuel-aware manoeuvre selection and independent safety verification produced measurable collision-risk reductions while maintaining compliance with assumed CubeSat mission constraints. These results provide encouraging evidence that the approach merits further investigation as a candidate architecture for future autonomous conjunction-management systems.